This fact sheet is for secondary school students undertaking work experience programs, and for their host employers.  It can be used as a tool to assist with task planning, and to develop appropriate induction, training and supervision to minimise any risk to students while they are ‘at work’.

Students must be given instruction, and must be supervised at all times.  Consideration must always be given to hazards when allocating tasks.  If any risk exists, students must NOT undertake the activity.

| ## Hazard | Possible Harmful Effects | Possible Employer Action to Prevent Injury / Illness | Preventative Action Students Can Take |
| --- | --- | --- | --- |
| Powered and non-powered tools | Electrocution Being struck by broken drill bits, cutters, swarf and offcuts (potential for serious eye, head or facial injury) Cuts and lacerations | Students must NOT operate powered tools - Provide instruction in safe use of hand tools | Students must NOT operate powered tools - Use hand tools carefully, according to your instructions |
| Working at heights | Falls from heights, potential for fatality or serious injury | Students must NOT work at heights where there is any risk of a fall | Students must NOT work at heights where there is any risk of a fall |
| Mobile plant including mobile platform | Falls from heights, potential for fatality or serious injury | Students must NOT operate mobile plant | Students must NOT operate mobile plant |
| Hazardous substances, e.g. plaster dust, asbestos, plasterboard fibres, medium density fibreboard (MDF) – formaldehyde vapour, wood adhesives | Even relatively short-term exposure can cause respiratory complaints Long-term dust exposures could lead to ‘chronic’ (ongoing) respiratory illnesses Formaldehyde is carcinogenic and exposure can lead to asbestosis and cancer | - Assess the work area prior to work including identification of materials with asbestos - Provide Material Safety Data Sheet (MSDS) and documented risk assessment for dangerous and hazardous chemicals - Provide dust collector bags for portable handtools - Provide personal protective equipment (PPE) to protect against inhalation of fine plaster dust | - Read MSDS and follow safe working instructions - Use PPE provided to protect you against dust and vapours |
| Manual handling (bending, reaching, stretching, pulling, lifting, repetitive motions, awkward postures, particularly when handling large plaster sheets or plastering overhead) | Musculoskeletal disorders | - Document and communicate safe manual handling procedures - Provide mechanical aids where practicable - Encourage team lifting where appropriate - Provide training in safe manual handling techniques | - Use mechanical aids provided - Seek help when you think a team lift is required - Exercise: warm up/stretch before starting work, and cool down/stretch at end of the shift or working day |
| Noise | Long-term exposure to excessive noise can lead to deafness | - Maintain powered tools to reduce noise - Limit noise exposure (e.g. by rotating jobs) - Provide PPE (hearing protection) | - Use PPE (hearing protection) provided |
| Hot weather conditions | Heat rashes, heat cramps, heat stress, dehydration, sunburn | - Provide regular rest breaks - Re-schedule work if extreme weather conditions present risk | - Dress appropriately for the conditions - Take scheduled breaks |
| Sexual harassment, work place bullying | Emotional stress, fear and anxiety, physical illness | - Establish work place policy - Provide staff briefings or training | - Report any concerns immediately |